5

DEMAND ASSESSMENT

# 5.1 	PHYSICAL INFRASTRUCTURE

# 5.1.1	WATER SUPPLY

# i) 	Existing Status

The protected water supply scheme for Allapuram Town Panchayat is the ‘Allapuram & Thorapadi Combined Water Supply Scheme’ maintained by Allapuram Town Panchayat. The head works is situated on the bank of River Palar at Kilmoonavoor village at a distance of 15 km from Allapuram town.  The water is pumped from the head works to Over head tanks and distributed to the town once in 20 days.  The general details of water supply in the town are given below:

General details – Water Supply

| Water Supply Source | Infiltration wells at Palar River |
| --- | --- |
| Name of the Scheme | Allapuram & Thorapadi Combined Water Supply Scheme (Head works – Kilmoonavoor village) |
| Population (2001) | 26,948 |
| Present population (2009) | 37,727 |
| Total number of wards | 18 |
| Total area of the town | 19 sq.km |
| Supply per day | 11 Ll |
| Supply per head | 29 lpcd |
| Frequency of water supply | Once in 20 days |
| Number of public fountains | 126 |
| Open wells | 14 |
| Hand pumps | 166 |
| Storage: |  |
| No. of OHTs and capacity | 4 Nos and 27 Ll |
| Distribution |  |
| Length of pumping mains | 15 km |
| Length of Distribution main | 20 km |
| Total number of assessments | 5879 |
| No. of HSCs | 3178 |
| Domestic connections | 3168 |
| Non Domestic connections | 10 |

Source: Allapuram Town Panchayat records

The water from Allapuram-Thorappadi Combined Drinking Water Supply Scheme, with five infiltration wells in the Palar is conveyed to a common collection well and distributed through the 6Ll overhead tank (OHT) at Thorappadi and four OHTs at Allapuram. Out of the 5 infiltration wells at Palar River only one is working at present. To meet the town’s water demand, the Town panchayat is purchasing 5 Ll once in two days from Vellore Corporation and supplying to all wards through  tanker lorries.

The town has 4 over head tanks and 2 ground level reservoirs. The table given below shows the locations and capacities of existing OHTs and ground level reservoirs in the town.

Table: 5.1 Details of existing OHTs

| Sl.No | Storage details | Capacity in Ll |
| --- | --- | --- |
| 1 | Ground Level Reservoir: |  |
|  | Vallalar Nagar | 1.00Ll |
|  | Rajiv Gandhi Nagar | 2.50Ll |
| 2 | Over Head Tanks |  |
|  | Rajiv Gandhi Nagar | 4.00Ll |
|  | Vallalar Nagar | 8.00Ll |
|  | Sastri Nagar | 12.00Ll |
|  | Chinna Allarpuram | 3.00Ll |
|  | Total Storage Capacity of OHTs | 27.00 Ll |

Source: Allapuram Town Panchayat records

ii)	Tariff details

The existing tariff and initial amount collected by the local body for individual connections are given in table below

Table 5.2 Water supply - Existing tariff

| HSCs | Tariff ( in Rs) | Initial amount (in Rs) |
| --- | --- | --- |
| Domestic | 50 | 5000 |
| Non Domestic | 100 | 10000 |
| Industrial | 100 | 10000 |

Source: Allapuram Town Panchayat Records

iii)	Performance Assessment

5.3 Water Supply Service Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Daily per capita supply | lpcd | 100 | 29 |
| 2 | Roads covered with distribution network | Percent | >100 | 83 |
| 3 | Storage Capacity with respect to supply | Percent | 50 | 245 |
| 4 | Property tax assessments covered by service connections | Percent | 85 | 54 |
| 5 | Proportion of non – domestic service connections | Percent | >5.00 | 0.3 |

Source: Analysis and Calculations

iv) 	Issues

The issues identified through discussions, field visits and service analysis are given below

- Inadequate source for supply.

- The per capita supply is 29 lpcd – Huge gap between the demand and supply.

- Huge demand- supply gap which is likely to widen drastically in future.

- Low coverage in terms of House service connections (54%)

- User charges not adequate to meet the annual operation and maintenance cost

- Inadequate staff for operation and maintenance

v)	Demand Assessment

Although, the total quantity of water supplied to the town is 11 Ll at a rate of 29 lpcd, the demand is met by the local sources available in the town. The demand for water supply in the intermediate and ultimate period is given in table below.

Table 5.4 Projection of water demand and supply

| Year | Population | Floating pop. | Existing supply in Ll | Floating pop. Demand at 20 lpcd (Ll) | Demand at 70 lpcd (Ll) | Demand at 100 lpcd (Ll) | Transmission loss at 15% (Ll) | Total demand at 100 lpcd (Ll) | Deficiency in Ll for 100 lpcd (Ll) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2009 | 37727 | 3700 | 11.00 | 0.74 | 27.15 | 37.73 | 5.66 | 44.13 | 33.13 |
| 2010 | 38727 | 3774 | 11.00 | 0.75 | 27.86 | 38.73 | 5.81 | 45.29 | 34.29 |
| 2015 | 44138 | 3849 | 11.00 | 0.77 | 31.67 | 44.14 | 6.62 | 51.53 | 40.53 |
| 2025 | 57335 | 9435 | 11.00 | 1.89 | 42.02 | 57.34 | 8.60 | 67.82 | 56.82 |
| 2040 | 84885 | 16511 | 11.00 | 3.30 | 62.72 | 84.89 | 12.73 | 100.92 | 89.92 |

Source: Analysis and calculation

Fig 5.1 Mettur Combined Water Supply Scheme

Demand at 70 lpcd: The present supply is inadequate. A new scheme for Allapuram & Thorapadi at the same source with head works at Iraiyankadu at a cost of Rs.392.00 lakhs is under progress by TWAD Board. After the completion of this scheme, the intermediate and ultimate stage water demand at 70 lpcd will be fulfilled. The existing storage capacity is 27 Ll and the demand for ultimate stage is 62.72 Ll. Therefore the storage requirement for intermediate period will be 42.02/2 = 21.01 < 27 Ll and for ultimate period will be 62.72/2 = 31.36 Ll > 27 Ll (existing). Hence, existing storage capacity is adequate only for the intermediate period with 70 lpcd supply.

Demand at 100 lpcd: The town is proposed to be annexed to Vellore Corporation limits by 2011.The prorata of water required to implement UGSS is 135 lpcd as per CPHEEO norms for towns with Water Supply and Sewerage system. The prorate of water demand considered for calculation is 100 lpcd. As the town has local sources (private and public), it is assumed that the rest of the water demand can be met by these local sources available in the town. The net future water demand at 100 lpcd for intermediate period is 67.82 Ll and ultimate period 100.92 Ll.

Allapuram Town Panchayat is included in the proposed CWSS with source from River Cauvery from Mettur Dam. The Detailed Project Report for this scheme is under preparation by TWAD Board. The additional water requirement at 100 lpcd supply will be met by the proposed scheme. The flow diagram of Mettur CWSS for Vellore district is shown in fig 3.1

Storage demand at 100 lpcd: The existing storage capacity is 27 Ll and the demand for ultimate stage is 100.92 Ll. Therefore, the storage requirement for intermediate period will be 67.82/2 = 33.91 > 27 Ll and for ultimate period will be 100.92/2 = 50.46Ll > 27 Ll (existing).

The service storage facility and the distribution system are not sufficient to meet the short-term demand. Thus, to meet intermediate demand at 100 lpcd supply, additional storage reservoirs of 7 Ll capacity and D-system of 8 km is needed.

vi.	Proposals by the consultant

The proportionate capital cost of the Mettur CWSS to Allapuram Town Panchayat is approximately Rs.3138.00 lakhs and the O&M cost is Rs.90.00 lakhs. In addition to the ongoing improvement scheme and Mettur CWSS, the projects identified for the water supply improvement includes improvement in distribution system and increasing the storage capacity to meet the future demand. The proposals for the water supply improvement are given below:

Table 5.5 Water supply – proposals by Consultant

| Sl.No | Project Description | Cost (Rs.in lakhs) |
| --- | --- | --- |
| 1 | Extension of D-system for uncovered areas (8 km) | 40.00 |
| 2 | Construction of additional storage reservoirs (7 Ll) | 30.00 |
|  | TOTAL | 70.00 |
|  | Annual Operation and Maintenance Cost | 3.50 |

Source: Analysis and discussion with town Panchayat officials

The total amount required for the implementation of the identified proposals under water supply improvement and Mettur CWSS is Rs.3208.00 lakhs.

5.1.2 	SEWERAGE & SANITATION

# i) 	Existing Status

The town does not have a Sewage System. The main mode of individual disposal in the town is through septic tanks, low cost sanitation units and through public / community toilets. The sullage is conveyed into the open storm water drains which drain into the PWD channels.

# ii) 	Demand Assessment

The town has 5879 assessments and about 6134 households. The total sewage output of ultimate population at 100 lpcd water supply will be 6.8 MLD. Therefore, it is suggested that Under Ground Sewerage for this town is essential. As consequence to upgradation as Corporation shortly, separate consultant need to be appointed for the preparation of UGSS DPR for the town panchayat.

# iii) 	Proposals by Consultant

For implementation of the UGSS, the investment requirement is calculated based on the generation of sewage output and treatment cost by the ultimate population, sewer lines, pumping mains and land required for STP.  The total cost is estimated as Rs.1920.00 lakhs and the O&M cost will be Rs.79.85 lakhs.

5.1.3	STORM WATER DRAINS

# i. 	Existing Status

The Town Panchayat has provided open storm water drains for a length of 18.585 km. The total length of the existing roads in the town is 25.98 km. The percentage coverage of storm water drains is about 72%. The classification of existing drains and its length is given in the table below.

Table 5.6 Existing drain details

| Sl.No | Type of drain | Length in km |
| --- | --- | --- |
| 1 | Pucca drains | 14.480 |
| 4 | Earthen drains | 4.105 |
|  | Total | 18.585 km |

# Source: Allapuram Town Panchayat Records

# ii)	Performance Assessment

Table 5.7 Storm water drains Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Road length covered with storm water drainage | Percent | 130 | 72 |
| 2 | Road length covered with pucca drains | Percent | 130 | 56 |

Source: Analysis and calculation

# iii) 	Issues

The issues identified through discussions, field visits and service analysis are given below:

- Inadequate coverage - Even though drains cover 72% of the road length, the coverage of pucca drains is only about 56%.

- Missing links: - Drains in some areas are not provided with adequate connections and network, thus not reaching the final point of disposal, which in turn gets stagnated in the low lying areas.

# iv)	Demand Assessment

It is essential to provide storm water drainage system within the town along all roads, connecting to channels or water bodies identified within the town. The runoff effectively captured will improve the quality and quantity of ground water. The existing and proposed drains need to be designed according to the topography. The missing drain links need to be developed and the damaged segments need to be rehabilitated. Therefore, development of missing drain links, up gradation of existing drains to pucca drains and provision of drains in newly developing areas are required for better drainage system.

# v)	Proposals

Proposals by consultant

The proposals identified for storm water drains are given below:

Table 5.8 Storm water drains - proposal by consultant

| Sl.No | Project Description | Cost (Rs.in lakhs) |
| --- | --- | --- |
| 1 | Upgradation of earthen drains to pucca drains (4.105 km) | 42.00 |
| 2 | Construction of new pucca drains (10 km) | 160.00 |
| 3 | Construction of new drains in the extension areas (5 km) | 80.00 |
| 4 | Construction of culverts at various sections (20 nos) | 15.00 |
|  | TOTAL | Rs.297.00 lakhs |

Source: Analysis and discussion with town Panchayat officials

Therefore the total amount required for the implementation of identified projects in storm water drains is Rs.297.00 lakhs.

5.1.4 	IMPROVEMENT TO WATER BODIES

i)	Existing Status

The town has PWD channels within its limits. The length of PWD channels in the town is about 4 km. The PWD channels in the town need to be strengthened, revamped and desilted for the entire length. The proposal is included in section 5.3 under “Projects to be implemented by other departments”.

5.1.5 	SOLID WASTE MANAGEMENT

i)	Existing Status

Allapuram town Panchayat generates 5.5 MT of solid waste per day. The Urban local body is responsible for collection, transportation and disposal of Solid Waste except untreated bio-medical waste and hazardous industrial waste. 80% of the generated waste is collected by the local body.

Primary Collection: The household waste produced is collected door to door in 15 wards by the SHGs and town Panchayat sanitary workers. 15 numbers of Tricycles are used for primary collection. A total of 34 sanitary workers are involved in the street sweeping and other waste collection activities.

Secondary Collection: The collected waste is transferred to the compost yard of 1.5 acres located at Sainathapuram through 1 tractor and 3 power tillers. The waste is segregated in the compost yard and the segregated waste is then allowed for aerobic decomposition. The manure (vermi compost) produced is sold at a cost of Rs.2 per kg.

Table 5.9 Solid waste – general details

| Quantity of waste generated | 5.5 MT |
| --- | --- |
| Quantity of waste collected | 4.4 MT |
| Number of workers available | 34 sanitary workers |
| Number of vehicles available | Power Tiller - 3 Tractor -1 Tricycles - 15 |
| Compost Yard | Compost is produced in the yard located at Sainathapuram - 1.5 Acres. |

Source: Allapuram Town Panchayat Records

ii)	Performance Assessment

Table 5.10 Solid Waste management Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Estimated waste generation | Grams / capita/ day | 210 | 145 |
| 2 | Waste collected as per ULB estimation (w.r.t. waste generation) | Percent | 100 | 80 |
| 3 | Door to door collection efficiency | Percent | 100 | 90 |

Source: Analysis and calculations

ii)	Issues

The issues identified through discussions, field visits and service analysis are given below:

- 100% collection efficiency is not achieved

- Of the 18 wards, 15 wards in the town are covered under door to door collection. In the rest 3 wards power tillers are used for collection.

- Absence of waste collection in slum areas

- Inadequate space in the compost yard for processing the entire waste collected.

iii)	Demand Assessment

The activities associated with the management of solid wastes from the point of generation to final disposal can be grouped into the following functional elements:

- Waste generation

- Waste handling, sorting and storing

- Collection

- Sorting, processing and transformation

- Transfer and transport

- Production of Compost

- Sale of compost

The inter-relationship between the functional elements is given in Fig 5.2

Fig 5.2	Functional Elements of Solid Waste Management System

The analysis of existing solid waste management practices in the town indicates that the town has structured primary and secondary collection arrangements. However, the disposal arrangements of the town need substantial improvement. The solid waste collected from only 3 wards (approximately 1 MT) is disposed at the compost yard. The solid waste collected from the other wards is dumped in the old dumping yard, which is located near the compost yard. The issues that need to be addressed are listed below:

- 100% door to door collection coverage

- Levy of user charges

- Waste minimization and resource recovery through sorting and recycling of materials

- Development and improvement of compost yard for the treatment of entire waste collected

- Development of regional landfill site

- Institutional strengthening and human resources development

iv) 	Proposals by Consultants

The Proposals for the improvement of solid waste management are given below

Table 5.11 Solid Waste management - proposal by consultant

| S.N | Project Description | Cost (Rs.in lakhs) |
| --- | --- | --- |
| 1 | Purchase of pushcarts with bins (8 nos) | 1.00 |
| 2 | Purchase of Hydraulic tipper lorry (2 nos) | 24.00 |
| 3 | Land cost for compost yard(2.2Acres) | 220.00 |
| 4 | Cost for Development of Compost yard | 240.00 |
| 5 | Proportionate cost for land fill site (2.223 Acres) | 223.00 |
| 6 | Cost for development of land fill site | 250.00 |
| 7 | IEC Activities | 5.00 |
|  | Total | 963.00 |
|  | Annual Operation and maintenance | 25.75 |

Source: Analysis, calculation and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under the Solid Waste Management will be Rs.963.00 lakhs.

5.1.6	ROADS

i) 	Existing Status:

Allapuram Town Panchayat is situated on the Cuddalore – Chitoor State Highway (SH-9). The other Major District Road in the town is Usoor – Vellore road. The details of the roads within the town limits are given in the table below.

Table 5.12 Category of roads

| Category | Length in km | % to total length |
| --- | --- | --- |
| Cuddalore – Chitoor State Highway | 1.25 | -- |
| Usoor – Vellore road | 0.75 | -- |
| Total length of other roads | 2.00 | -- |
| Town Panchayat Roads |  |  |
| CC Roads | 7.20 | 30.0% |
| BT Roads | 9.45 | 39.4% |
| Earthen Road / WBM | 7.33 | 30.6% |
| Total Length of Town Panchayat Roads | 23.98 | 100% |
| Total length of roads within the town | 25.98 |  |

Source: Allapuram Town Panchayat records

ii)	Performance Assessment

Table 5.13 Roads - Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Road density | km/sq.km | 10-15 | 1.40 |
| 2 | Per capita road length | meters | 1.50 | 0.70 |
| 3 | Proportion of surfaced roads | Percent | 100 | 69.0 |

Source: Analysis and calculation

iii)	Issues:

The issues identified through discussions, field visits and analysis are given below:

  - The WBM and Earthen roads constitute 30% of total road length.

  - Absence of roads at extension areas

  - Internal circulation roads being covered with top-soil and mud reducing the driving safety.

iv) 	Demand Assessment

The circulation network forms basic network of the town to move, to interact, and interlink the residential areas, the work places, and other activity centers. Allapuram has good connectivity with urban centers like Vellore, Tiruvannamalai, Katpadi, Arcot and Arani.

The town has 30% un-surfaced roads, and the internal un-surfaced road network need to be upgraded to BT and CC roads. The newly developed areas/extension areas in the town need to be provided with circulation roads. In addition to this, all the existing BT roads have to be periodically resurfaced. To regulate the illegal parking, traffic markings to show parking areas, no parking areas, pedestrian crossings, etc need to be done.

v)	Proposals by Consultants

The proposals for the improvement of roads are given below:

Table 5.14 Roads – proposal by consultant

| S.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Upgradation of earthen road to CC roads (4.33 km) | 109.00 |
| 2 | Upgradation of earthen road to BT roads (3 km) | 69.00 |
| 3 | Formation of new BT roads at extension areas (2 km) | 46.00 |
| 4 | Formation of new CC roads at extension areas (3 km) | 75.00 |
| 5 | Relaying of BT roads (10 km) | 150.00 |
| 6 | Maintenance of existing CC roads (7.20 km) | 15.00 |
| 7 | Provision of road markings and signages | 1.00 |
|  | TOTAL | 465.00 |
|  | Annual Operation and Maintenance | 15.00 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under Roads will be Rs.465.00 lakhs.

5.1.7 	BUS STAND

The town has no bus stand within its limits. The town is served by bus stands located at Vellore Corporation.

5.1.8 	BUS SHELTERS

i) 	Existing Status:

The town has 8 bus stops within limits. All the bus stops have bus shelters, which are in need of repairs and maintenance work. The list of bus stops in the town is as follows

- Sankaranpalayam

- College road

- Sainathapuram

- Kuppam

- Collector  Bungalow

- Toll Gate

- Periya  Allapuram (2 nos)

ii)	Demand Assessment

The bus shelters can be provided with information boards, seating facility and space for advertisement. Two of the existing bus shelters have been provided with advertisement space and income is generated. Towards repairs and maintenance of bus shelters and provision of advertisement space etc, the total cost proposed is Rs.4.00 lakhs.

5.1.9	STREET LIGHTS

i. 	Existing status

The town is lit by 804 street lights of which 66.70% of lights constitute tube lights. The break-up details of the street lights in the town are as follows:

Table 5.15 Existing street lights and energy consumption

| S.No | Existing Light details | Numbers | Percentage distribution |  |
| --- | --- | --- | --- | --- |
| 1 | Tube lights (40w) | 537 | 66.70 |  |
| 2 | Sodium Vapour lamp (250w) | 210 | 26.10 |  |
| 3 | High Mast lights | 5 | 0.63 |  |
| 4 | Compact Florescent Light | 52 | 6.47 |  |
|  | Total | 804 |  |  |
|  | Per hour consumption | 97365 W |  |
|  |  | 97.635 kW/hr |  |
|  | No. of timer switches required | 40 (18 nos available) |  |

Source: ULB records, Analysis and calculations

Therefore, the present level of annual energy consumption of street lights in Allapuram Town Panchayat is 416040.65 KW. i.e. Rs 14.5 lakhs per year on an average.

ii)	Performance Assessment

Table 5.16 Street Lights - Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Spacing between lamp posts | meters | 30 | 32 |
| 2 | Proportion of fluorescent lamps (tube lights) w.r.t. total fixtures | Percent | ≥80 | 67 |
| 3 | Proportion of high power fixtures w.r.t. total fixtures | Percent | ≤20 | 27 |

Source: Analysis and calculation

iii)	Demand Assessment

The standard distance between two lights as per norms is 30 m. The existing street lights in the town are distributed at 32 m gap on an average. The newly developed areas in the town need to be provided with adequate street lights. In addition to this, energy saving measures such as retrofitting of tube lights with T5 and CFL lights and, installation of timer switches are recommended.

iv)	Proposals by Consultants

The proposals for the improvement of street lights are given below:

Table 5.17 Street lights - proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of new lights (T5) with poles at internal roads (60 nos) | 8.00 |
| 2 | Provision of new lights with poles at extension areas (T5 - 150 nos) | 19.00 |
| 4 | Installation of timer switches (22 nos) | 1.50 |
| 5 | Retrofitting of existing tube lights with T5 lights (537 nos) | 5.50 |
|  | TOTAL | 34.00 |
|  | Annual Operation and Maintenance cost | 1.70 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under street lights is Rs.34.00 lakhs.

5.2	SOCIAL INFRASTRUCTURE

5.2.1	SLUM IMPROVEMENT

i)	Existing Status

The town has 6 notified slums with a population of 7000, which is 19% of the total town’s population. The details of the existing slums are given in the table below:

Table 5.18 Existing status - Slum

| Sl.No. | Name | Status |
| --- | --- | --- |
| 1 | Chinna Allapuram | Notified |
| 2 | Sastri Nagar | Notified |
| 3 | Periyar Nagar | Notified |
| 4 | Sivaraj Nagar | Notified |
| 5 | Muslim Colony | Notified |
| 6 | Bagadhur Shah Nagar | Notified |

Source: Town Panchayat records

Accessibility to facilities

The slums of Allapuram Town Panchayat have accessibility to Panchayat union schools and the Government schools. There are noon meal centers in the town supporting children from LIG and EWS groups. The slums also have good accessibility to health sub- centers and private health facilities. The internal circulation has been provided with cement concrete roads at some stretches and most of the roads are earthen roads. The toilet facility is absent in most of the houses and the people are depending on the community toilets.

ii)	Demand Assessment

Slum improvement consists of physical, social, economic, organizational and environmental improvements undertaken cooperatively and locally among citizens, community groups, businesses and local authorities. Based on the per capita cost and number of additional new houses, the cost required for improvement of slums is assessed. The cost required for improvement of slums is Rs.1380.00 lakhs.

    - PARKS AND PLAY FIELDS

# i) 	Existing Status

The town has a park named Anna park at Sastri Nagar (area – 1 acre) maintained by the Town Panchayat. The issues identified in the park are absence of compound wall, seating, greeneries, water and lighting facilities.

ii)	Demand Assessment

Parks are the major contributors to the physical and aesthetic quality of urban neighborhoods. It will also improve the ambient environment quality of the town. The parks in the town should be provided with water supply, sanitation facilities, lighting facilities and landscaping. Recreational facilities like play materials also need to be installed. The improvement works to the existing park areas and maintenance on a sustainable basis is essential. The private sectors, especially the SHGs could be approached for maintenance of parks.

The town has layout/new developments at extension areas. The open space reservation areas in these layouts needs to be identified, demarcated, notified and maintained under parks and open spaces.

The proposals for the improvement of parks are given below:

Table 5.19 Parks - proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of landscaping, seating arrangements and landscape elements | 15.00 |
| 2 | Provision of compound wall, water and lighting facilities | 10.00 |
| 3 | Provision of watchman cabin, entrance gate | 2.00 |
|  | TOTAL | Rs.27.00 lakhs |
|  | Annual Operation and maintenance | 0.81 |

Source: Analysis and discussion with town Panchayat officials

Therefore, the proposed capital cost towards the improvement of parks and playgrounds is estimated as Rs.27.00 lakhs.

5.2.3	MARKET

There is no market in this town at present. The market demand for the town is met by the private shops available at various locations of the town and an Uzhavar shandy located at Usoor road. As the town is located near Vellore, the markets in Vellore serve Allapuram for marketing purposes. The Town Panchayat has no vacant land suitable for market.

5.2.4 	PUBLIC CONVENIENCE

i)	Existing Status

The town has 5 public conveniences at various locations. The location of public conveniences facilities are given in the table below:

Table 5.20 Public Conveniences

| Sl.No | Location | Remarks |
| --- | --- | --- |
| 1 | Sastri Nagar | Damaged buildings and absence of water facilities |
| 2 | Periyar Nagar |  |
| 3 | Muslim colony |  |
| 4 | Chinna Allarpuram | Well maintained |
| 5 | Kamarajar Nagar | Well maintained |

Source: Allapuram Town Panchayat records

ii) 	Proposals by Consultant

The proposals under the improvement of public conveniences are given below:

Table 5.21 Public convenience - proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Reconstruction of public conveniences at Sastri Nagar, Periyar Nagar and Muslim colony | 18.00 |
| 2 | Maintenance and repair works to 2 existing toilets | 6.00 |
|  | TOTAL | 24.00 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under public conveniences section is Rs. 24.00 lakhs

5.2.5	BURIAL GROUNDS

i)	Existing status

The town has 3 community burial grounds maintained by the town panchayat and a community maintained burial ground within its limits. The locations of the burial grounds are given below

- Chinna Allapuram

- Sainathapuram

- Kamarajar Colony

- Usoor Road (Christian)

Inadequate basic facilities such as water supply, compound wall, lighting, drains, absence of burning sheds, absence of watchman shed and toilet facilities are the issues identified in the burial grounds.

ii)	Demand Assessment

The burial grounds need to be provided with compound wall with good access road, circulation road, well with pumping motor and a storage tank, proper shelters at the entry points and burning huts, special tree plantation with flowering, ornamental and shady species, water taps, bathing and toilet facilities, shelter for the people accompanying the bodies, a maintenance cell and a shed for security staff.

iii)	Proposal by Consultant

The proposals under the improvement of burial grounds are given below:

Table 5.22 Burial Ground - proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of gasifier Crematorium in Chinna Allapuram Burial ground | 50.00 |
| 2 | Provision of compound wall for 3 burial grounds | 9.00 |
| 3 | Provision of infrastructure facilities at 3 burial grounds | 15.00 |
| 4 | Provision of landscaping, circulation roads and security shed | 20.00 |
|  | TOTAL | 94.00 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under burial ground improvement is Rs.94.00 lakhs.

5.2.7	PROPERTY MAPPING & AUTOMATED BILLING MACHINES

Property taxes are the major source of revenue of ULBs in India. However, in most cities, not all properties are assessed, records are not updated regularly, and tax collection is far from satisfactory. ULBs can increase revenue by improving property inventories, record management, public awareness and participation, and tax collection.

As a first step, the Town Panchayat may initiate a door to door survey of all existing properties, with details on size & built up area of the property, present use, number of households, houses, size of families, etc. which would form a database for GIS projects. At the rate of Rs.30.00 per property (including secondary works), the estimated cost for the project would be Rs.2.00 lakhs. This would also aid in identifying unassessed and under assessed properties, rationalization of property tax etc.

To facilitate easy and timely collection of Property tax, Profession Tax, WS charges and UGSS charges an automatic hand held billing machines are proposed at a cost of Rs.1.20 lakhs.

  - PROJECTS TO BE IMPLEMENTED BY OTHER DEPARTMENTS

The list of identified projects to be implemented by other departments is given below

Table 5.23 Proposals to be implemented by other Agencies

| Sl.No | Project description | Implementing Agency | Cost (Rs. in lakhs) |
| --- | --- | --- | --- |
| 1 | Desilting, Revamping and strengthening of PWD channel | Water Resources Department - PWD | 100.00 |
| 2 | Widening Of State Highway | NH Department | 150.00 |
| 3 | Improvement of Panchayat Union school buildings and provision of essential furniture and infrastructure facilities | Panchayat union / Education Department | 100.00 |
| 4 | Awareness campaign for NGOs on their roles and responsibilities in community development | Town Panchayat | 2.00 |
| 5 | Provision of public / community health facilities in the town | Public health department / Panchayat Union/ Vellore Corporation | - |

Source: Analysis and discussion with town Panchayat officials

  - TOWN INVESTMENT PLAN

The Town Investment Plan for Allapuram Town Panchayat under TIP has been detailed out in the previous sections.  The projects include major infrastructural projects initiated by the State government recently for physical and social infrastructure projects by way of meeting the long felt needs to provide comfortable living environment to the citizens of the town. The proposals are segregated as short term, medium and long term projects.

In all, 12 projects are proposed of which 7 relate to provision of physical infrastructural facilities and, the rest relate to social infrastructure and commercial ventures. The summary of capital investment is given in the table below:

Table 5.24 Town investment plan

| S.N | Physical Infrastructure facilities | Amount ( in lakhs) | Phasing of Investments |  |  |  |  | Medium term Investment 2020 | Long term Investment 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 |  |  |
| 1 | Water Supply | 3208.00 |  |  |  |  |  | 1690.00 | 2344.00 |
|  | Cost required for water supply improvement | 70.00 | 70.00 |  |  |  |  |  |  |
|  | Cost for Mettur Combined Water Supply Scheme | 3138.00 | 1882.80 | 1255.20 |  |  |  |  |  |
| 2 | Under Ground sewerage System | 1920.00 |  |  |  |  |  | 2444.00 | 2741.00 |
|  | Cost required for UGSS with treatment plant | 1597.00 |  | 996.00 | 601.00 |  |  |  |  |
|  | Land cost for treatment plant (3.22acres) | 323.00 | 323.00 |  |  |  |  |  |  |
| 3 | Storm Water Drains | 297.00 |  | 75.00 | 125.00 | 97.00 |  | 2548.00 | 2850.00 |
| 4 | Solid Waste Management | 963.00 |  |  |  |  |  | 531.00 | 1720.00 |
|  | Primary, secondary collection improvement | 25.00 | 25.00 |  |  |  |  |  |  |
|  | ICE Activities | 5.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |  |  |
|  | Land cost for compost yard (2.2 Acres) | 220.00 | 220.00 |  |  |  |  |  |  |
|  | Development cost for compost yard | 240.00 |  | 240.00 |  |  |  |  |  |
|  | Proportionate Land cost for regional land fill site( 2.223) | 223.00 |  |  |  | 223.00 |  |  |  |
|  | Proportionate Development cost for regional Landfill site | 250.00 |  |  |  |  | 250.00 |  |  |
| 5 | Roads | 465.00 |  |  |  |  |  | 2240.00 | 2741.00 |
|  | Upgradation, formation of new roads and improvements | 300.00 |  |  | 178.00 | 122.00 |  |  |  |
|  | Relaying of BT roads & maintenance of CC roads | 165.00 |  |  |  | 125.00 | 40.00 |  |  |
| 6 | Bus Shelter | 4.00 | 4.00 |  |  |  |  | As per demand |  |
| 7 | Street Lights | 34.00 |  |  |  |  |  | 362.00 | 438.00 |
|  | Provision of additional lights | 27.00 |  |  | 13.00 | 14.00 |  |  |  |
|  | Installation of timer switches (22 nos) | 1.50 | 1.50 |  |  |  |  |  |  |
|  | Retrofitting existing lights & energy saving devices | 5.50 | 1.00 | 1.25 | 1.75 | 1.50 |  |  |  |
| 8 | Slum Improvement | 1380.00 |  | 483.00 | 621.00 | 276.00 |  | 400.00 | 900.00 |
| 9 | Parks | 27.00 | 27.00 |  |  |  |  | As per Demand |  |
| 10 | Burial Ground | 94.00 |  |  |  |  |  | As per Demand |  |
|  | Basic facilities in Burial Ground | 44.00 | 20.00 | 24.00 |  |  |  |  |  |
|  | Provision of Gasifier crematorium | 50.00 | 50.00 |  |  |  |  |  |  |
| 11 | Public convenience | 24.00 | 24.00 |  |  |  |  | As per Demand |  |
| 12 | Property mapping and automatic billing machines | 3.20 | 3.20 |  |  |  |  |  |  |
|  | Total | 8419.20 | 2652.50 | 3075.45 | 1540.75 | 859.50 | 291.00 | 10215.00 | 13734.00 |