5

CAPITAL INVESTMENT NEEDS

# 5.1	PHYSICAL INFRASTRUCTURE

# 5.1.1	WATER SUPPLY

# i) 	Existing Status

The main sources of water supply to Chengam Town Panchayat are Cheyyar River (infiltration wells) and the open wells & bore wells located at various locations of the town. The water supply scheme for Chengam with head works on Cheyyar River bank at Valayampattu Road was executed by TWAD Board (year 2001).  The following table furnishes the general details of water supply in Chengam Town Panchayat.

General details – Water Supply

| Water Supply Source | - Infiltration wells at Melapallayam (Valayampattu Road) - 2 - Open wells -12 - Bore wells - 2 |
| --- | --- |
| Population (2001) | 23,223 |
| Present population(2009) | 26,000 |
| Total number of wards | 18 |
| Total area of the town | 8 sq.km |
| Supply per day | 17.62 Ll |
| Supply per head | 67 lpcd |
| Frequency of water supply | Daily |
| Number of public fountains | 158 |
| Hand pumps | 93 |
| Mini power pumps | 21 |
| Storage |  |
| No. of OHTs and capacity | 13 Nos and 12.12 Ll |
| Distribution: |  |
| Length of Distribution main | 21.94 km |
| Hours of pumping at head works | 24 hrs |
| Total no. of assessments | 6700 |
| No. of HSC’s | 1734 |
| Domestic connections | 1705 |
| Non Domestic connections | 29 |

Source: Town Panchayat Records

ii) 	Storage

The town has 13 Over Head Tanks through which the water is supplied to 18 wards. The location, capacity and number of fillings details are given in the table below

Table 5.1 Existing OHT details

| S.No | Location of OHTs | Capacity in Ll | No of fillings (per day) | Supply form OHTs (Ll) |
| --- | --- | --- | --- | --- |
| 1 | Town Panchayat office campus | 7.00 | 1 ½ | 10.50 |
| 2 | Keil Annaimangalam (near School) | 0.10 | 1 | 0.10 |
| 3 | Mel Annaimangalam | 0.02 | 1 | 0.02 |
| 4 | Kamaraj Nagar (near Sangam Theatre) | 1.00 | 2 | 2.00 |
| 5 | Krishnapuram Nehru Nagar | 0.30 | 1 | 0.30 |
| 6 | Krishnapuram 2nd Street | 0.30 | 1 | 0.30 |
| 7 | Indira nagar (opp. To temple) | 0.10 | 1 | 0.10 |
| 8 | Milath nagar old Polur road | 1.00 | 1 | 1.00 |
| 9 | Thokavadi Playground | 1.00 | 2 | 2.00 |
| 10 | Nadu Annaimangalam | 0.30 | 1 | 0.30 |
| 11 | Maligapuram (near Primary Health center) | 0.60 | 1 | 0.60 |
| 12 | Thiruvalluvar Nagar youth Welfare association | 0.30 | 1 | 0.30 |
| 13 | Narayanapuram | 0.10 | 1 | 0.10 |
|  | Total Storage Capacity | 12.12 Ll |  | 17.62 LL |

Source: Town Panchayat Records

In addition to the infiltration wells there are open wells and bore wells which supply drinking water to the town. The list of open wells and bore wells is given in the table below

Table 5.2 Existing well details

| S.No | Location of wells | Pump set details |
| --- | --- | --- |
| 1 | Pachayamman Koil – Opposite to Cheyyar River (well 1) | 3 HP |
| 2 | Pachayamman Koil – Opposite to Cheyyar River (well 2) | 3 HP and 2 HP |
| 3 | Thalavanayakkan pettai – Cheyyar River basin | 3 HP and 2 HP |
| 4 | Melapalayam – near old pump house | 3 HP |
| 5 | Melapalayam | 3 HP |
| 6 | Krishnapuram Hamlet | 2 HP |
| 7 | Thokkavadi Eri | 2 HP |
| 8 | Thokavadi (at Cheyyar River) | 3 HP and 7.5 HP |
| 9 | RajaThangal Eri – Anaimangalam | 2 HP |
| 10 | Varadhanthangal Eri – Thiruvalluvar Nagar | 2 HP – 2 Nos |
| 11 | Keil Anaimangalam | 2 HP |
| 12 | Chengam Cheyyar Maadhapillai Thoppu well | 5 HP |
| 13 | Narayanapuram Bore well | 2 HP |
| 14 | Indira Nagar | 2 HP |

Source: Town Panchayat Records

iii)	Tariff details

The existing tariff and initial amount collected by the local body for individual connections is given in table below :

Table 5.3 Water supply - Existing tariff

| HSCs | Tariff ( in Rs) | Initial amount (in Rs) |
| --- | --- | --- |
| Domestic | 40 | 3000 |
| Non Domestic | 100 | 5000 |
| Industrial | 100 | 6000 |

Source: Chengam Town Panchayat records

# iv)	Performance Assessment

5.4 Water Supply Service Indicators

| S.N | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Daily per capita supply | lpcd | 70 | 67 |
| 2 | Roads covered with distribution network | Percent | >100 | 97 |
| 3 | Storage Capacity with respect to supply | Percent | 50 | 68.7 |
| 4 | Property tax assessments covered by service connections | Percent | 85 | 26 |
| 5 | Proportion of non – domestic service connections | Percent | >5.00 | 1.6 |

Source: Analysis and Calculations.

# v)	Demand Assessment

The total quantity of water supplied to the town is 17.62 Ll at a rate of 67 lpcd. The demands for water supply in the intermediate and ultimate period are 25.61 Ll and 29.93 Ll respectively and the details are given in the table below:

Table 5.5 Projection of water demand and supply

| Year | Population | Floating population | Existing Quantity of supply (in Ll) | Water demand in Ll at 70 lpcd | Transmission loss at 15% | Floating population demand at 20 lpcd | Total Demand | Deficiency in Ll |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2009 | 26000 | 2000 | 17.62 | 18.20 | 2.73 | 0.040 | 20.97 | 3.35 |
| 2010 | 26423 | 2000 | 17.62 | 18.50 | 2.77 | 0.040 | 21.31 | 3.69 |
| 2025 | 31756 | 2700 | 17.62 | 22.23 | 3.33 | 0.054 | 25.61 | 7.99 |
| 2040 | 37089 | 3800 | 17.62 | 25.96 | 3.89 | 0.076 | 29.93 | 12.31 |

Source: Analysis and calculation

The storage capacity of the service reservoirs is of 12.12 Ll and the demand for ultimate stage is 29.93 Ll. Therefore the storage requirement will be 25.61/2 = 12.80 Ll > 12.12 Ll (existing) for intermediate stage and 29.93/2 = 14.97 >12.12 Ll (existing) for ultimate stage. Therefore, the town does not need additional storage requirement for the intermediate period.

Water Supply Improvement Scheme for Chengam, Pudupalayam Town Panchayats and the villages under both Panchayat Unions is proposed by TWAD Board and has got Administrative Sanction as per GO no.97 dated 15-6-09. The intermediate and ultimate water demand can be met by the implementation of the improvement scheme proposed by TWAD. The total cost of the scheme is Rs.888.00 lakhs and the operation and maintenance cost is Rs.24.50 lakhs. The proportionate cost for Chengam Town Panchayat is Rs.501.40 lakhs with an annual O&M of Rs.14.02 lakhs.

vi.	Proposals by the consultant

In addition to the proposed water supply improvement scheme, the projects identified for water supply improvement are given below:

Table 5.6 Water supply – proposals

| Sl. No | Description | Rs. in lakhs |
| --- | --- | --- |
| 1 | Laying of new Distribution lines (2 km) | 10.00 |
| 2 | Treatment plant (sedimentation) at Pachayamman Koil at existing source | 20.00 |
|  | TOTAL | 30.00 |

Source: Analysis and discussion with town Panchayat officials

Therefore, the total amount required for the implementation of identified projects in water supply improvement (including the proposed improvement scheme) is Rs.531.40 lakhs and the total annual operation and maintenance cost will be Rs.26.57 lakhs.

5.1.2 	SEWERAGE

# i) 	Existing Status

The town does not have Sewerage System. The main mode of individual disposal in the town is through septic tanks, low cost sanitation units and through public / community toilets. The sullage is conveyed into the open drains which drain into the River, Eri and PWD channels.

# ii)   	Issues

- Easy access to open lands led to open defecation and unsanitary conditions.

- Discharge of untreated sullage into water bodies via storm water drains and natural drains resulting in degradation of the environment.

- Absence of diversion points and treatment facilities at disposal points in Cheyyar River

- Lack of awareness of public on the importance of proper sanitary arrangements & disposal mechanisms.

- Pollution to water bodies due to activities like washing, bathing etc.

# iii)	Demand Assessment

The Town Panchayat has not identified any projects for the improvement of sanitation facilities in the town. A Detailed Project Report for Integrated sewerage scheme for the Chengam Town Panchayat needs to be prepared by a WS & Sanitation consultant. The total sewage output is 1.8 MLD at present and 2.5 MLD for ultimate population (calculated at the rate of 70 lpcd supply).

# iv) 	Proposals by Consultant

Although conventional sewage systems are being proposed in many urban local bodies in Tamil Nadu, it is a capital intensive and expensive in O&M and treatment costs. Also, conventional sewage systems (UGSS) transport excreta away from the toilet user, they fail to contain and sanitize, instead releasing pathogens and nutrients into the downstream environment.

These systems mix faeces, urine, flush water and toilet paper with grey water, storm water and industrial effluents, usually overtaxing the design capacity of the treatment plants. Very few urban local bodies are able to afford fully functional sewage systems.

Considering the socio – economic and fiscal character of Chengam Town Panchayat, Decentralized Waste Water Treatment Systems (DEWATS) system is proposed. DEWATS is an innovative, partly standardized synthesis of well known WW treatment modules. It is based on mechanical & biological (anaerobic, aerobic) process. The salient features of the DEWATS system is briefly described as below:

- Waste Water treatment components that are especially suitable for tropical conditions.

- Low Operation cost and low energy bills.

- Low skill requirements, thereby

- Recycling of water from system which can be used for irrigation, landscapes or flushing purposes.

- iBio gas produced successfully used in heating, cooking and lighting.

CRITERIA FOR DEWATS

Minimum O&M

Affordable

Minimum energy Consumption

Simple

Long lasting

Reclamation of minerals from domestic waste as sludge used for composting.

Fig 5.1 Technical Options for DEWATS.

Treated waste water

Sludge

Biogas

DEWATS Plant

Laundry

Toilets/ bathroom

Kitchen

Other source

Waste water

Treatment plant

Re-use

Irrigation, flushing

Cooking Fuel/ Lighting

Compost

Fig 5.2 Principle of DEWATS

The approximate cost for providing DEWATs for Chengam Town Panchayat may be about Rs.350.00 lakhs and the Operation and maintenance cost will be Rs.10.50 lakhs.

5.1.3	STORM WATER DRAINS

i)	Existing Status

The Town Panchayat has provided open storm water drains for a length of 23.39 km. The total length of the existing roads in the town is 22.66 km. The percentage coverage of storm water drains is about 103%. The classification of existing drains and its length is given in the table below.

Table 5.7 Existing drain details

| Sl.No | Type of drain | Length in km |
| --- | --- | --- |
| 1 | Pucca drains | 19.964 |
| 2 | Kutcha drains | 3.426 |
|  | Total | 23.390 km |

# ii)	Performance Assessment

Table 5.8 Storm water drains Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Road length covered with storm water drainage | Percent | 130 | 103 |
| 2 | Road length covered with pucca drains | Percent | 130 | 88 |

# iii)	Issues

The issues identified through discussions, field visits and service analysis is; the waste water generated in the town is disposed at Cheyyar River without treatment at disposal points. There are about 8 disposal points in Cheyyar River. They are Melapalayam, Durgai Amman Koil Street, Thalavanayakkan pettai, nearby Ramakrishna School, Quaid-e-Milleth Street, MGR Nagar, Thokkavadi and Millath Nagar.

# iv)	Demand Assessment

The town has 103% of drain coverage of which pucca drains covers 88% of roads. The kutcha drains need to be upgraded into pucca drains. The town does not have water stagnation problem during monsoons due to adequate drains and availability of open lands. The runoff can be effectively captured in the water bodies and the wells distributed throughout the town area, subsequently improving the quality and quantity of ground water, which is the main drinking source to the town.

# v)	Proposals

Proposals by consultant

The proposals for storm water drain improvement is given in the table below

Table 5.9 Storm water drains - proposal by consultant

| SL.No | Project Description | Cost (Rs.in lakhs) |
| --- | --- | --- |
| 1 | Upgradation of existing Kutcha drains to pucca drains (3.5 km) | 35.00 |
| 2 | Construction of new pucca drains at internal roads (6 km) | 96.00 |
| 3 | Construction of new drains along State Highways (10 km) | 160.00 |
| 4 | Construction of culverts at various sections (40 nos) | 60.00 |
|  | TOTAL | Rs 351.00 lakhs |
|  | Annual operation and maintenance cost | 17.55 |

Source: Analysis and discussion with town Panchayat officials

Therefore the total amount required for the implementation of identified projects in storm water drains is Rs.351.00 lakhs.

5.1.4 	IMPROVEMENT TO WATER BODIES

Cheyyar River, the main drinking water source runs through the town in east- west direction. The town also has 3 number of PWD owned lakes namely Thokkavadi lake, Chengam lake, and Varadhanthangal lake. The ULB owned water body is located opposite bus stand area.

ii)	Demand Assessment

The water bodies need to be protected and strengthened for various reasons such as

- Recharge of ground water table,

- To maintain quantity and quality of source wells,

- Alleviate future flooding problems.

As the PWD channels are the main water conveying source for irrigation in the town, the existing channels in the town need to be strengthened, revamped and desilted periodically for the entire length. In addition to this, the inlet and outlets of the lakes need to be repaired and maintained. Towards conservation of water bodies, construction of embankment, desilting, fencing on lakes and cleaning, removal of surface growths in Cheyyar River is proposed. The proposed projects need to be taken by the PWD department, which is added in section 5.3. The Proposals identified for the improvement of town Panchayat owned water body located opposite to bus stand are desilting, strengthening of bunds, fencing around the pond at a total cost of Rs.20.00 Lakhs.

5.1.5 	SOLID WASTE MANAGEMENT

i)	Existing Status

Chengam Town Panchayat generates 3 MT of solid waste per day. The Urban local body is responsible for collection, transportation and disposal of Solid Waste except untreated bio-medical waste and hazardous industrial waste.100% of the generated waste is collected by the local body.

Primary Collection: The household waste produced is collected door-to- door in all wards by the SHGs and Town Panchayat sanitary workers. 20 number of push carts are used for door to door collection. A total of 38 sanitary workers including SHGs are involved in the street sweeping and other waste collection activities.

Secondary Collection:  The collected waste is transferred to the compost yard of 1.47 acres at Thalavanayakkan Pettai, located at Chengam - Paramananthal road through 1 tractor and 1 mini lorry. The waste is segregated at the compost and the segregated waste is then allowed for aerobic decomposition.

Table 5.10 Solid waste – general details

| Quantity of waste generated | 3 MT |
| --- | --- |
| Quantity of waste collected | 2.4 MT |
| Number of workers available | 20 sanitary workers 18 Self Help Group members |
| Number of vehicles available | Minilorry - 1 Tractor - 1 Pushcart - 20 |
| Compost Yard | Thalavanayakan Pettai with area of 1.47 Acres. |

Source: Town Panchayat Records

ii)	Performance Assessment

Table 5.11 Solid Waste management Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Estimated waste generation | Grams / capita/ day | 210 | 90 |
| 2 | Waste collected as per ULB estimation (w.r.t. waste generation) | Percent | 100 | 80 |
| 3 | Door to door collection efficiency | Percent | 100 | 80 |

Source: Analysis and Calculations

iii) 	Issues

There are some hamlet areas not covered under door to door collection namely Krishnapuram, Nehru Nagar, Thokkavadi, Thiruvallur Nagar and Anai Mangalam. Due to the availability of the open lands and fields, these settlements dispose degradable waste in their own back yards. The non degradable and recyclable waste is collected once in a week in these areas.

iv)  	Demand Assessment

The activities associated with the management of solid wastes from the point of generation to final disposal can be grouped into the following functional elements:

- Waste generation

- Waste handling and sorting, storing

- Collection

- Sorting, processing and transformation

- Transfer and transport

- Production of Compost

- Sale of compost

The inter-relationship between the functional elements is given in Fig 5.3

Fig 5.3	Functional Elements of Solid Waste Management System

The reason for low per capita waste generation is most of the houses located near the fields dispose the waste in their backyard and fields.

v) 	Projects Identified by Consultants

The Town Panchayat has proposed for the improvement of facilities in compost yard, provision of bore well, compound wall and gate at a cost of Rs.2.40 lakhs. In addition to this the projects identified for the improvement of solid waste management are given below

Table 5.12 Solid Waste management - proposal by consultant

| S.N | Project Description | Cost (Rs.in lakhs) |
| --- | --- | --- |
| 1 | Purchase of pushcarts with bins (27 nos) | 3.00 |
| 2 | Purchase of 1 tractor | 7.00 |
| 3 | Purchase of 1 Tipper lorry | 12.00 |
|  | Land Cost required for compost yard (1.13 acres) | 35.00 |
|  | Cost required for development of composite yard (1.13 acres) | 100.00 |
| 4 | Land required for solid waste management (0.61 Acres) | 20 |
| 5. | Cost required for improvement of solid waste management | 100.00 |
|  | TOTAL | 277.00 |
|  | Annual Operation and Maintenance | 13.8 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under the Solid Waste Management will be Rs.279.40 lakhs.

5.1.6 	ROADS

i) 	Existing Status:

Chengam is situated on the Krishnagiri - Puducherry National Highway (NH-66).  The details of the roads within the town limits are given in the table below.

Table 5.13 Category of roads

| Category | Length in km | % to total length |
| --- | --- | --- |
| Krishnagiri - Pondicherry National Highway | 5.00 | -- |
| Chengam-Polur State Highway | 8.00 | -- |
| Total Length of Other roads | 13.00 | -- |
| Town Panchayat Roads |  |  |
| CC Roads | 13.220 | 58.3% |
| BT Roads | 0.545 | 2.4% |
| Earthen Road / WBM | 8.899 | 39.2% |
| Total Length of Town Panchayat Roads | 22.664 | 100% |
| Total road length within the town | 35.664 | -- |

Source: Chengam Town Panchayat

ii)	Performance Assessment

Table 5.14 Performance Indicators - roads

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Road density | km/sq.km | 10-15 | 5.18 |
| 2 | Per capita road length | meters | 1.75 | 1.25 |
| 3 | Proportion of surfaced roads | Percent | 100 | 60.7 |

Source: Analysis and Calculations

iii)	Issues:

The issues identified through discussions, field visits and service analysis are given below:

  - The WBM and Earthen roads constitute 40% of total road length.

  - Absence of roads at extension areas and hamlet areas

  - Traffic congestion during peak hours at core area

  - The road margins in the core area are encroached by street vendors, illegal parking and other informal activities.

  - Internal circulation roads being covered with sand and soil reducing driving safety.

iv) 	Demand Assessment

The circulation network forms basic network of the town to move, to interact, and interlink the residentials, the work places, and other activity centers. As Chengam is situated on Krishnagiri - Puducherry National Highway, the town has good connectivity to urban centers like Chennai, Tiruvannamalai, Vellore, Arani, Krishnagiri and Bengaluru.

The town has 40% un-surfaced roads, and the internal un-surfaced road network need be upgraded to BT and CC. Bye pass road connecting Thanthai Periyar Nagar and Bus stand need to be developed to ease congestion in the core area. The newly developing areas/extension areas in the town need to be provided with surfaced roads. In addition to this, all the existing BT roads have to be periodically resurfaced. To regulate the illegal parking, specific parking areas need to be provided in Polur road and Bus stand area. Traffic markings such as parking, no parking, pedestrian crossing, school zone etc are needed.

v)	Proposals by Consultants

The proposals for the improvement of roads are given below

Table 5.15 Roads – proposal by consultant

| S.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Upgradation of earthen / WBM road to BT roads (9 km) | 208.00 |
| 2 | Formation of new BT roads at extension areas (5 km) | 115.00 |
| 3 | Relaying of roads (0.6 km) | 9.00 |
| 4 | Maintenance of existing CC roads (13.22 km) | 27.00 |
| 5 | Provision of road markings and signages | 1.00 |
|  | TOTAL | 360.00 |
|  | Annual Operation and Maintenance | 18.00 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under Roads will be Rs.360.00 Lakhs.

5.1.7 BUS STAND

i) 	Existing Status:

The town has C class bus stand located on Polur road, maintained by the Town Panchayat. Absence of bus bays, CC flooring are the issues identified in bus stand. The bus stand has shops (38 nos) owned by the Town Panchayat and the income generated from shops is Rs.18 lakhs per annum and from entry fees Rs.4.8 lakhs per annum. The Town Panchayat has proposed improvements in the existing bus stand at a cost of 25.50 which is under progress.

ii)	Demand Assessment

In addition to the above, the proposals for improvement of bus stand are

- Provision of CC flooring for rest of the area in bus stand

- Provision for advertisement boards

Table 5.17 Bus stand - proposal by Consultant

| S.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of CC flooring (other than drive way) | 20.00 |
| 2 | Provision of advertisement spaces | 1.00 |
|  | TOTAL | 21.00 |
|  | Annual Operation and Maintenance | 0.63 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under Bus stand is Rs.21.00 lakhs.

5.1.8 	BUS SHELTERS

i) 	Existing Status:

The town has four bus stops within the town limits. The location and status of bus shelters are given in the table below:

Table 5.17 Existing status – Bus shelters

| S.No | Location of Bus stops | Availability of Bus shelter | Remarks |
| --- | --- | --- | --- |
| 1 | Chengam to Polur road | No | -- |
| 2 | Chengam to Kuppanatham Dam | No | -- |
| 3 | Chengam to Tiruvannamalai | One side available | Well maintained |
| 4 | Chengam to Bengaluru | No | -- |

Source: Town Panchayat records

ii)	Demand Assessment

The bus stops without bus shelters need to be provided with bus shelters. The bus shelters can be provided with information boards, seating facility and space for advertisement which will generate additional revenue.

iii)	Proposals by Consultants

The projects identified under the improvement of bus shelters are given below

Table 5.18 Bus Shelter - proposal by consultant

| S.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of bus shelters at Polur road and Chengam-Bengaluru road (4 nos) | 6.00 |
| 2 | Provision of bus shelters at Kuppanatham dam stop and Chengam-Tiruvannamalai stop (3 nos) | 4.50 |
|  | TOTAL | Rs.10.50 lakhs |
|  | Annual Operation and Maintenance | 0.32 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under Bus shelter is Rs.10.50 lakhs.

5.1.9	STREET LIGHTS

i. 	Existing status

The town is illuminated by 749 street lights of which 70% of lights constitute tube light. The break-up details of the street lights in the town are as follows:

Table 5.19 Existing street lights and energy consumption

| S.No | Existing Light details | Numbers | Percentage distribution | Consumption in w |
| --- | --- | --- | --- | --- |
| 1 | Tube lights (40w) | 520 | 69.40 | 28600 |
| 2 | Sodium Vapour lamp (250w) | 195 | 26.03 | 54990 |
| 3 | Mercury lights | 7 | 1.00 | 19874 |
| 4 | High Mast lights | 2 | 0.23 | 2820 |
|  | Compact Florescent Light | 25 | 3.34 | 750 |
|  | Total | 749 | 100% | 89134 |
|  | Per hour consumption | 89134 W |  |  |
|  |  | 89.134 kW/hr |  |  |
|  | No of control switches required | 38 (4 nos available) |  |  |

Source: ULB records, Analysis and calculations

Therefore, the present level of annual energy consumption of street lights in Chengam Town Panchayat is 380869.58 KW. i.e. Rs.13.33 lakhs per year on an average.

ii)	Performance Assessment

Table 5.20 Street Light - Indicators

| Sl.No | Service Indicators | Unit | Normative Standard | Current Status |
| --- | --- | --- | --- | --- |
| 1 | Spacing between lamp posts | meters | 30 | 55 |
| 2 | Proportion of fluorescent lamps (tube lights) w.r.t. total fixtures | Percent | ≥80 | 69 |
| 3 | Proportion of high power fixtures w.r.t. total fixtures | Percent | ≤20 | 27 |

Source: Analysis and Calculations

iii)	Demand Assessment

The standard distance between two lights as per norms is 30 m. The existing street lights in the town are distributed at 55 m gap in average. Thus, new lights need to be provided for better illumination. The newly developed areas and the highway roads linking the hamlets also need to be provided with adequate street lights. In addition to this, energy saving measures such as retrofitting of tube lights with T5 and T8 lights, installation of timer switches is recommended.

iv)	Proposals by Consultants

The projects identified for the improvement of street lights is given below:

Table 5.21 Street lights - proposal by consultant

| S.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of new lights with poles at Other roads including Highways (T8 - 350 nos) | 45.50 |
| 2 | Provision of new lights with poles at internal roads (T5 - 280 nos) | 34.00 |
| 3 | Provision of new lights with poles at newly developing areas (T5 - 100 nos) | 12.00 |
| 4 | High Mast lights at Bus Stand (1 no) | 7.00 |
| 5 | Mini mast Light at Thokkavadi (1 no) - Thiruvalluvar Nagar (1 no) - Polur – Kuppanatham road junction(1 no) - Polur – Perumal koil street junction(1 no) - at Millath Nagar (2 nos) | 21.00 |
| 6 | Installation of timer switches (34 nos) | 2.50 |
| 7 | Retrofitting of existing tube lights with T5 lights (527 nos) | 5.50 |
|  | TOTAL | 127.50 |
|  | Annual Operation and Maintenance | 6.38 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under street lights is Rs.127.50 Lakhs.

5.2	SOCIAL INFRASTRUCTURE

5.2.1	SLUM IMPROVEMENT

i)	Existing Status

The town has 3 notified slums with a total population of 3000 which is 9% of the total town population. The details of the existing slums are given in the table below

Table 5.22 Existing details - Slum

| Sl.No. | Name | Status |
| --- | --- | --- |
| 1 | Thokkavadi | Notified |
| 2 | Anaimangalam | Notified |
| 3 | Krishnapuram | Notified |

Source: Town Panchayat records

Accessibility to facilities

The slums of Chengam Town Panchayat have accessibility to Panchayat union schools and the Government schools. There are noon meal centers in the town supporting the children from LIG and EWS. The slums also have good accessibility to health sub centers and private health facilities available in the town. The internal circulation has been facilitated by cement concrete roads except few areas. Individual toilet facilities are inadequate increasing the dependency of most of the households on the community toilets.

ii)	Demand Assessment

Slum improvement consists of physical, social, economic, organizational and environmental improvements undertaken co-operatively and locally among citizens, community groups, businesses and local authorities. The slums need to be provide with surface access roads. Based on the per capita cost and number of additional new houses, the cost required for improvement of slum is assessed. The total cost required will be Rs.280.00 lakhs.

5.2.2 	PARKS AND PLAY FIELDS

i) 	Existing Status

The town has 2 parks maintained by the Town Panchayat. The existing status of the parks and playgrounds are given below:

Table 5.23   Existing park areas in the town

| Sl.No | Park/open space | Remarks |
| --- | --- | --- |
| 1 | Children Park (Opposite to bus stand) | Provided with basic facilities, compound wall and pathways. Absence of landscaping |
| 2 | Park at Pumping station | Compound wall and pathways available. Absence of landscaping |

Sources: From ULB Records

ii)	Demand Assessment

Parks are the major contributors to the physical and aesthetic quality of urban neighborhoods. It will also improve the ambient environment quality of the town. The parks in the town should be provided with water supply sanitation facilities, Lighting facilities and landscaping. Recreational facilities like play materials also need to be installed. The improvement works to the existing park areas and maintenance on a sustainable basis is essential. The private sectors, especially the SHGs could be approached for maintenance of parks. A new park at Millath nagar near Polur-Kuppanathur Dam junction is proposed by the Ulb (50 cents).

iii) 	Proposals by Consultant

The proposals for the improvement of parks is given below:

Table 5.24 Parks – proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of landscaping, seating arrangements and repair works at Children park | 2.00 |
| 2 | Provision of landscaping at the park at pumping station | 2.00 |
| 3 | Development of park at Millath nagar near Polur-Kuppanathur Dam junction | 25.00 |
|  | TOTAL | Rs.29.00 lakhs |

Source: Analysis and discussion with Town Panchayat officials

Therefore the proposed capital cost towards the improvement of parks is estimated as Rs.29.00 lakhs.

5.2.3	MARKET

i)	Existing Status

The town has one weekly market (1 acre) at Thukkapettai and Daily market at Polur road (near police station).

Weekly market:

The weekly market is organized twice in a week (for vegetable and for cow/goat). The weekly market has provided with three sides compound wall, toilet and water facilities.

Daily market:

The daily market has only 12 permanent shops and others are temporary sheds. The market need to be reconstructed in a planned manner, facilitating easy access to all shops with good internal circulation, loading /unloading facilities and parking.

The annual income from daily market is Rs.40000 and weekly market is Rs.3.00 lakhs. In addition to this, the market demand of the town is also served by Uzhavar Shandy located at Chengam - Polur road (opposite to Taluk Office).

ii) 	Proposals by Consultant

As the town serves as a prime marketing centre for the surrounding hinterland, improvement in marketing centres would inturn generate revenue to the town. The details are proposed as below:

- Demolition and reconstruction of daily market.

- Improvement of facilities in weekly market

The proposals under the improvement of market are given below:

Table 5.25 Market - proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
|  | Daily Market: |  |
| 1 | Reconstruction of Daily market with 50 Nos. RCC roof stalls | 40.00 |
| 2 | Repair of existing infrastructure facilities in daily market | 5.00 |
| 3 | Improvement of toilets and Provision of lighting arrangements | 2.00 |
|  | Sub Total | 47.00 |
|  | Weekly Market |  |
| 4 | Construction of internal circulation roads | 15.00 |
| 5 | Provision of Lighting and basic facilities | 10.00 |
|  | Sub Total | 25.00 |
|  | TOTAL | 72.00 |
|  | Annual O&M | 2.16 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under market improvement is Rs.72.00 lakhs.

5.2.4 	PUBLIC CONVENIENCE:

The town has 1 sanitary complex and 2 pay and use toilets. The existing public conveniences are well maintained and the location of the sanitary facilities are given in the table below:

Table 5.26 Sanitary facilities

| Sl.No | Sanitary facility | Number of seats | Location |
| --- | --- | --- | --- |
| 1 | Sanitary Complex | .. | Thokkavadi |
| 2 | Pay and use toilet | 10 | Bus stand |
| 3 | Pay and use toilet (new) | 10 | Bus stand |

Source: Town Panchayat records

The public conveniences are well maintained so there is no further proposal for public convenience.

5.2.5 	SLAUGHTER HOUSE

The town has 2 number of slaughter houses separately for goat and cow. The annual income generated from goat slaughter house is Rs. 24,900 and from cow slaughter is Rs.2700.

5.2.6	BURIAL GROUNDS

i)	Existing status

The town has 11 community burial grounds within its limits.  Inadequate basic facilities such as water supply, compound wall, lighting, drains, absence of burning sheds, absence of watchman shed and toilet facilities are the issues identified in the burial grounds. The location of burial grounds is given below:

- Millath Nagar

- Melapalayam

- Thokkavadi

- Thukkapettai (Muslim – 2 nos)

- Millath Nagar (Muslim)

- Krishnapuram

- Nehru Nagar

- Anai Mangalam

- Thukkapettai (Christian)

- Thukkapettai

ii)	Demand Assessment

The burial grounds need to be provided with compound wall with good access road, circulation road, well with pumping motor and a storage tank, proper shelters at the entry points and burning sheds, special tree plantation with flowering, ornamental and shady species, water taps, bathing and toilet facilities, shelter for the people accompanying the bodies, a maintenance cell and a shed for security staff.

iii)	Proposal by Consultant

The proposals under the improvement of burial grounds are given below:

Table 5.27 Burial Ground - proposal by consultant

| Sl.No | Project Description | Cost (Rs in lakhs) |
| --- | --- | --- |
| 1 | Provision of compound wall for 7 burial grounds | 12.00 |
| 2 | Provision of infrastructure facilities to all burial grounds | 22.00 |
| 3 | Provision of landscaping, circulation paths and security shed | 11.00 |
|  | TOTAL | 45.00 |
|  | Annual Operation and Maintenance | 1.35 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under burial ground improvement is Rs.45.00 lakhs.

5.2.7	REMUNERATIVE PROJECTS

The remunerative projects identified in the town are given below

Table 5.28 Remunerative proposals

| Sl.No | Project description | Cost (Rs. in lakhs) |
| --- | --- | --- |
| 1 | Construction of Commercial building (for office) at first floor of old bus stand shopping complex | 35.00 |
| 2 | Construction of Community hall at weekly market | 50.00 |
| 3 | Construction of 20 shops near existing public convenience at Bus stand | 25.00 |
| 4 | Construction of 10 shops by demolishing temporary flower shop sheds at bus stand | 20.00 |
| 5 | Construction of two wheeler parking stand at bus stand | 10.00 |
|  | TOTAL | Rs.140.00 lakhs |
|  | Annual Operation and Maintenance | 4.20 |

Source: Analysis and discussion with town Panchayat officials

The total cost required for the implementation of the projects identified under remunerative projects is Rs.140.00 lakhs.

5.2.8	OTHER PROJECTS

The other projects identified in the town are given below

- Construction of new Town Panchayat office inside the existing office campus – Rs.30.00 lakhs.

The total cost required for the implementation of the projects identified under other projects is Rs.30.00 lakhs.

5.2.9	PROPERTY MAPPING & AUTOMATED BILLING MACHINES

Property taxes are the major source of revenue of ULBs in India. However, in most cities, not all properties are assessed, records are not updated regularly, and tax collection is far from satisfactory. ULBs can increase revenue by improving property inventories, record management, public awareness and participation, and tax collection.

As a first step, the ULB may initiate a door to door survey of all existing properties, with details on size & built up area of the property, present use, number of households, houses, size of families, etc. which would form a database for GIS projects. At the rate of Rs.30.00 per property (including the secondary works), the estimated cost for the project would be Rs.3.00 lakhs. This would also aid in identifying unassessed and under assessed properties, rationalization of property tax etc.

To facilitate easy and timely collection of Property tax, Profession Tax, WS charges and UGSS charges an automatic hand held billing machines are proposed at a cost of Rs.1.20 lakhs.

5.3   PROJECTS TO BE IMPLEMENTED BY OTHER DEPARTMENT

The list of identified projects to be implemented by other departments is given below

Table 5.29 Proposals to be implemented by other Agencies

| S.No | Project description | Implementing Agency | Cost (Rs. in lakhs) |
| --- | --- | --- | --- |
| 1 | Widening of Chengam Polur Highway (2km) | NH | 60.00 |
| 2 | Construction of Bridge across Cheyyar River at Thokkavadi, ThalavanayakkanPettai and MGR Nagar (3 nos) | Water Resources Department – PWD / Town Panchayat | 250.00 |
| 3 | Demarcation, fencing and Construction of Embankment at Varadhanthangal lake | Water Resources Department - PWD | 100.00 |
| 4 | Demarcation, fencing and Construction of Embankment at Thokkavadi lake | Water Resources Department - PWD | 100.00 |
| 5 | Demarcation, fencing and Construction of Embankment at Chengam lake | Water Resources Department - PWD | 150.00 |
| 6 | Cleaning and removal of surface growth in Cheyyar River | Water Resources Department - PWD | 5 |
| 7 | Cleaning and improving PWD channels | Water Resources Department - PWD | 50.00 |
| 8 | Development of Bye pass road from Millath Nagar Bridge to Anjeneyar Temple connecting Thanthai Periyar Nagar and Bus stand (3 km) | N.H.A | 515.00 |
| 9 | Provision of public / community health facilities in the town | Public health department / Panchayat Union | - |

Source: Analysis and discussion with town Panchayat officials

5.4   TOWN INVESTMENT PLAN

The Capital Investment needs for Chengam Town Panchayat has been detailed out in the previous sections.  The projects include major infrastructural projects initiated by the State government recently for physical and social infrastructure projects by way of meeting the long felt needs to provide comfortable living environment to the citizens of the town. The proposals are segregated as short term, medium and long term projects.

In all, 19 projects are proposed of which 9 relate to provision of physical infrastructural facilities and, the rest relate to social infrastructure and commercial ventures. The summary of capital investment is given in the table below:

Table 5.30 Summary of Capital Investments

| S.n | Physical Infrastructure facilities | Amount | Phasing of Investments |  |  |  |  | Medium term Investment 2020 | Long term Investment 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ( in lakhs) | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 |  |  |
| 1 | Water Supply | 531.40 |  |  |  |  |  | 1050.00 | 600.00 |
|  | Chengam water Supply Improvement Scheme | 501.40 |  | 200.00 | 301.00 |  |  |  |  |
|  | Laying of new distribution lines and treatment facilities | 30.00 | 30.00 |  |  |  |  |  |  |
| 2 | Sewerage System | 350.00 |  | 350.00 |  |  |  | 570.00 | 600.00 |
| 3 | Storm Water Drains | 351.00 |  |  | 66.00 | 125.00 | 160.00 | 780.00 | 460.00 |
| 4 | Improvement of Water Bodies | 20.00 | 20.00 |  |  |  |  | 50.00 | 100.00 |
| 5 | Solid Waste Management | 279.40 |  |  |  |  |  | 1200.00 | 2400.00 |
|  | Existing compost yard Improvement | 2.40 | 2.40 |  |  |  |  |  |  |
|  | Primary, secondary collection improvement | 22.00 | 22.00 |  |  |  |  |  |  |
|  | Land cost for Compost yard (1.31 Acers) | 35.00 | 35.00 |  |  |  |  |  |  |
|  | Compost yard improvement (1.31 acres) | 100.00 |  | 100.00 |  |  |  |  |  |
|  | Proportionate cost for regional Landfill site (0.62acre) | 20.00 |  |  | 20.00 |  |  |  |  |
|  | Development of regional land fill site | 100.00 |  |  |  | 100.00 |  |  |  |
| 6 | Roads | 360.00 |  |  |  |  |  | 220.00 | 450.00 |
|  | Upgaradation, formation of new roads and improvement to road geometry | 324.00 |  |  | 126.00 | 120.00 | 78.00 |  |  |
|  | Relaying and maintenance of existing roads | 36.00 |  |  |  | 36.00 |  |  |  |
| 7 | Bus Stand | 21.00 | 21.00 |  |  |  |  | As per demand |  |
| 8 | Bus Shelter | 10.50 | 10.50 |  |  |  |  | As per demand |  |
| 9 | Street Lights | 127.50 |  |  |  |  |  | 20.00 | 75.00 |
|  | Provision of additional lights | 91.50 | 18.00 | 23.00 | 25.00 | 25.50 |  |  |  |
|  | High mast light and UG cables | 7.00 | 7.00 |  |  |  |  |  |  |
|  | Mini Mast light and UG cables | 21.00 |  | 21.00 |  |  |  |  |  |
|  | Retrofitting existing lights & energy saving devices | 8.00 |  | 3.00 | 2.50 | 2.50 |  |  |  |
| 9 | Slum Improvement | 280.00 |  | 50.00 | 110.00 | 120.00 |  | 100.00 | 260.00 |
| 10 | Parks | 29.00 | 4.00 | 25.00 |  |  |  | As per demand |  |
| 11 | Market | 72.00 |  |  |  |  |  | As per demand |  |
|  | Reconstruction of daily market | 47.00 | 47.00 |  |  |  |  | As per demand |  |
|  | Improvement to weekly market | 25.00 |  | 25.00 |  |  |  |  |  |
| 12 | Burial Ground Improvements | 45.00 | 45.00 |  |  |  |  | As per demand |  |
| 13 | Property mapping & automatic billing machine | 4.20 | 4.20 |  |  |  |  | As per demand |  |
| 14 | Other projects | 170.00 |  |  |  |  |  | As per demand |  |
|  | Construction of new Town Panchayat office inside the existing office campus | 30.00 | 30.00 |  |  |  |  |  |  |
|  | Construction of Commercial building (for office) at first floor of old bus stand shopping complex | 35.00 | 35.00 |  |  |  |  |  |  |
|  | Construction of Community hall at weekly market | 50.00 | 50.00 |  |  |  |  |  |  |
|  | Construction of 20 shops near existing public convenience at Bus stand | 25.00 |  | 25.00 |  |  |  |  |  |
|  | Construction of 10 shops by demolishing temporary flower shop sheds at bus stand | 20.00 |  | 20.00 |  |  |  |  |  |
|  | Construction of two wheeler parking stand at bus stand | 10.00 | 10.00 |  |  |  |  |  |  |
|  | Total | 2651.00 | 391.10 | 842.40 | 650.50 | 529.00 | 238.00 | 3990.00 | 4945.00 |