ANNEXURE 4

LONG TERM INVESTMENT REQUIREMENT

To assess the long term investment requirement in each sector of infrastructure, the existing levels of key services is benchmarked against their norms and standards. The norms and standards are based on benchmarks, UDPFI guidelines, best practices etc. The future demand for physical and social infrastructure calculated is given in the table below

| Description | Units | 2010 | 2020 | 2025 | 2040 |
| --- | --- | --- | --- | --- | --- |
| Population | Nos | 26423 | 29978 | 31756 | 37089 |
| Water Supply |  |  |  |  |  |
| Per Capita supply @ 135 liters | ML | 3.57 | 4.05 | 4.29 | 5.01 |
| Requirement of Distribution line @ >100% to road length | km | 40 | 45 | 48 | 56 |
| Under Ground Sewerage System |  |  |  |  |  |
| Network coverage @ 100% to road length | km | 40 | 45 | 48 | 56 |
| Storm Water Drains |  |  |  |  |  |
| Requirement @ 130% to road length | km | 52 | 58 | 62 | 72 |
| Solid Waste Management |  |  |  |  |  |
| Waste generation at 210 gms | MT | 5.55 | 6.30 | 6.67 | 7.79 |
| Compost yard area requirement @ 1 acre per 10000 population | Acres | 3 | 3 | 3 | 4 |
| Roads |  |  |  |  |  |
| Road requirement with benchmark @ 1.5 to 2 m per capita road length | km | 39.63 | 44.97 | 47.63 | 55.63 |
| Street Lights |  |  |  |  |  |
| Requirement @ 30 m gap between street lights | Nos | 1321 | 1499 | 1588 | 1854 |
| Parks and Open spaces |  |  |  |  |  |
| Requirement @1.0 to 1.2 hectare per 1000 population | Acres | 65.26 | 74.05 | 78.44 | 91.61 |
| Community Hall and Library |  |  |  |  |  |
| Requirement @ 1 per 15000 population (2000 sq.m) | Nos | 2 | 2 | 2 | 2 |